## Mathematical Framework and Quantitative Image Analysis Methodology in PhasePix

### 1. Spatial Calibration and Photometric Preprocessing

#### 1.1. Automated Scale-Bar Detection and Spatial Calibration

To convert discrete pixel coordinates into physical units (μm), PhasePix isolates the upper-right quadrant of the SEM information banner (x∈0.63W,W-5, y∈H-hfooter+3,H-0.48hfooter) and applies morphological opening using a horizontal rectangular structuring element (15×1 px) on thresholded binary layers (I>90, with a fallback at I>70). The pixel span Lbar,px between the outermost vertical tick marks defines the spatial calibration factor:

- Purpose: Conversion of pixel dimensions to physical micrometres (μm/px).

Rμm/px=Lbar,μmLbar,px, Rpx/μm=1Rμm/px

Where:

Rμm/px — spatial calibration ratio (μm⋅pixel-1);

Lbar,μm — nominal physical length of the SEM scale bar (μm);

Lbar,px — detected or manually verified horizontal pixel span of the scale bar (pixels);

Rpx/μm — spatial resolution (pixels⋅μm-1).

#### 1.2. Multi-Pass Segmentation and Shadow-Recovery Preprocessing

For multi-phase whipped emulsions, segmentation is executed via a hierarchical three-pass architecture on the footer-cropped micrograph (H'=H-hfooter):

- Pass 1 (Dual-Pass Air Bubble Detection & Dark-Crater Fallback): Contrast Limited Adaptive Histogram Equalization (CLAHE; clip limit 2.0, tile grid 8×8) is applied to generate a direct image ICLAHE and an inverted image Iinv=255-ICLAHE. Both are evaluated by the Cellpose neural network at a target geometric mean diameter:

dtarget,air=clipmaxdmin,air,px,8⋅maxdmax,air,px,15, 25, 220

Candidate masks from the inverted pass are retained only if their median intensity is darker than the global image median (medianIk<medianI) and their intersection-over-union (IoU) with direct-pass masks is ≤0.40. Deep shadowed pores missed by neural flow fields are recovered via Gaussian smoothing (9×9), thresholding at the 22nd intensity percentile (Iblur≤P22), morphological opening/closing (5×5 ellipse), and filtering by circularity (Φ≥0.35) and size (deq,px∈0.85dmin,air,px,1.15dmax,air,px).

- Pass 2 (Matrix Fat Globule Segmentation): Executed on the CLAHE-enhanced micrograph using a fat-optimized target diameter:

dtarget,fat=clip1.25⋅dmin,fat,μmRμm/px, 15, 45

- Pass 3 (Localized Crater Deep-Scan for Adsorbed Fat): For each confirmed air bubble bounding box (padded by 8 px), internal pixel intensities Ibub are linearly stretched between the 2nd and 98th percentiles, processed with adaptive CLAHE (clip limit 3.5), and subjected to non-linear gamma shadow lifting:

Istretch=clipI-P2IbubmaxP98Ibub-P2Ibub,1)×255, 0, 255, Iγ=255⋅Istretch2550.85

Candidate globules detected inside the crater are clipped strictly to the bubble interior mask and accepted if their overlap with existing fat masks is <30%.

#### 1.3. Topological De-duplication ("Matryoshka" Containment Filter)

To eliminate nested double-detections across multi-scale passes, objects are sorted in descending order of area (A1≥A2≥…≥AM). For each object k, its external contour is flood-filled to yield a solid binary footprint Ωfilled,k (accounting for annular/donut-like crater rims). An object is rejected as a nested artifact if its containment fraction within the cumulative footprint Ωocc of larger accepted objects exceeds threshold τnest:

Ωfilled,k∩ΩoccΩfilled,k≥τnest

where

τnest=0.50 during Pass 1 air-bubble filtering,

τnest=0.60 during raw mask filtering,

τnest=0.45 during final cross-pass air-bubble deduplication.

Furthermore, in Two-Pass mode, any Pass-2 object with deq>dmax,fat overlapping the cumulative air bubble mask by ≥35% is rejected as a duplicate crater floor.

1.4. Automated Optical & Morphological Diagnostics (AI Parameter Advisor) To assist users in selecting optimal segmentation thresholds, PhasePix includes a deterministic pre-analysis diagnostic engine. This engine evaluates the micrograph's optical properties to recommend baseline parameters, which the user can manually accept or override:

- Optical Contrast & Background Analysis: The algorithm applies Gaussian smoothing and evaluates intensity percentiles (e.g., 5th and 95th) to determine the global optical contrast and baseline background intensity. Low-contrast images trigger recommendations for higher neural network sensitivity (more negative cellprob_threshold).

- Initial Feature Sizing: Using morphological white top-hat filtering (for bright matrix fat) and localized thresholding (for dark craters), the engine estimates the initial size distribution (10th to 95th percentiles) of the dispersed phases. These estimates are used to recommend the physical Min and Max size boundaries.

- Model Selection Logic: The system recommends the baseline cyto2 model for standard, high-contrast micrographs, and switches the recommendation to the super-generalist cyto3 model if low contrast, deep shadows, or significant internal crater speckle (indicating hidden adsorbed fat) are detected.

### 2. Primary Morphological Descriptors

For every segmented object i consisting of Npx,i foreground pixels, the following morphological metrics are computed:

#### 2.1. Projected Object Area (Ai)

- Purpose: Quantifies the 2D cross-sectional area of a bubble or fat globule in physical units.

Ai=Npx,i⋅Rμm/px2

Where:

Ai — projected area of object i (μm2);

Npx,i — number of pixels in the binary mask of object i (pixels);

Rμm/px — spatial calibration factor (μm⋅pixel-1).

#### 2.2. Equivalent Circular Diameter (deq,i)

- Purpose: Defines the diameter of a circle possessing the same projected area as the segmented object.

- Formula:

deq,i=4Aiπ

- Where:

  - deq,i — area-equivalent diameter of object i (μm);

  - Ai — projected area of object i (μm2).

#### 2.3. Circularity Shape Factor (Φi)

- Purpose: Measures object roundness and boundary regularity to separate spherical droplets/pores from fracture artifacts (Φi=1.0 for a perfect circle; Φi<1.0 for irregular or elongated shapes).

Φi=4πNpx,iPpx,i2

Where:

Φi — dimensionless circularity factor of object i;

Npx,i — pixel area of object i (pixels2);

Ppx,i — contour perimeter of object i in pixels (pixels).

#### 2.4. Fitted Ellipse Axes (Lmajor,i,Lminor,i) and Aspect Ratio (ARi)

- Purpose: Quantifies particle elongation and provides the visible interfacial chord length Lc for adsorbed globules.

Lmajor,i=lmajor,px,i⋅Rμm/px, Lminor,i=lminor,px,i⋅Rμm/px, ARi=Lmajor,imaxLminor,i,10-6

Where:

lmajor,px,i,lminor,px,i — major and minor axis lengths of the ellipse that has the same normalized second central moments as the object region (or fitted via direct least-squares cv2.fitEllipse for manual contours) (pixels);

Lmajor,i,Lminor,i — physical major and minor axis lengths (μm);

ARi — dimensionless aspect ratio (ARi≥1.0).

#### 2.5. Solidity / Neck Concavity Index (Si)

- Purpose: Measures the convexity of a particle or sintered doublet; lower solidity indicates pronounced concave necks formed during partial coalescence of fat globules.

Si=Npx,iAhull,px,i

Where:

Si — dimensionless solidity (Si∈(0,1]);

Npx,i — actual pixel area of object i;

Ahull,px,i — pixel area of the smallest convex polygon (convex hull) enclosing object i.

- Contour Regularization and Human-in-the-Loop Curve Fitting

#### 3.1. Periodic Cubic B-Spline Interpolation (Exact Smooth Curve Mode)

- Purpose: Constructs a C2-continuous smooth closed boundary passing exactly through user-clicked vertices p0,p1,…,pK-1 without AI edge-snapping distortion.

Vertices are ordered angularly around their centroid by αk=atan2yk-y,xk-x and closed by setting pK=p0. Using normalized cumulative chord-length parameterization tk∈0,1:

t0=0, tk=m=1kpm-pm-12m=1Kpm-pm-12

piecewise cubic polynomials Sxt and Syt are solved under periodic boundary conditions:

Stk=pk, S'0=S'1, S''0=S''1

Where:

pk=xk,yk — k-th user-defined boundary control point (pixels);

tk — normalized arc-length parameter (tk∈0,1);

Sxt,Syt — periodic cubic spline interpolants evaluated at M=160 uniform nodes.

#### 3.2. Discrete Fourier Contour Regularization

- Purpose: Eliminates pixelation staircase artifacts and high-frequency jaggedness from automated and SAM-assisted masks while preserving low-order ovoid pore geometry.

- The closed contour is resampled at N uniform arc-length steps into a complex sequence zn=xn+jyn (n=0,…,N-1, where j=-1) and transformed via the Discrete Fourier Transform (DFT):

Cm=n=0N-1zne-j2πmnN, Cm=Cm,if m≤Kh or m≥N-Kh0,otherwise, zn=1Nm=0N-1Cmej2πmnN

Where:

zn,zn — raw and Fourier-smoothed complex contour coordinates at node n;

Cm,Cm — raw and low-pass filtered complex Fourier descriptors at frequency index m;

Kh — number of retained low-frequency harmonics (Kh=6 for small circular objects, Kh=8 for large irregular pores);

N — total resampled contour vertices (N=160).

### 3.3. Interactive AI-Assisted Manual Correction For human-in-the-loop corrections, PhasePix utilizes localized neural networks to fit exact physical boundaries to user inputs:

- AI Smart Box (SAM integration): When a user draws a bounding box around a particle, the Segment Anything Model (SAM) is prompted using the geometric center of the box as a positive foreground point and the inset corners of the box as negative background points. The optimal mask is hole-filled, morphologically opened to separate touching particles, and smoothed using low-order Discrete Fourier Transform.

- Single-Click Center Detection: When a user clicks the center of a pore, PhasePix evaluates a localized region of interest around the coordinate using Cellpose across a geometric progression of target diameters. If neural segmentation is insufficient, it falls back to a 36-ray polar search to detect the maximum radial brightness gradient (the crater wall), fitting a smooth parametric ellipse to the detected boundary points.

### 4. Granulometry and Particle Size Distribution (PSD) Metrics

To prevent systematic underestimation of matrix fat globule size by partially submerged interfacial globules, all objects classified as adsorbed_fat are strictly excluded from matrix PSD calculations. For a given phase containing n valid objects with equivalent diameters d1,d2,…,dn:

#### 4.1. Sauter Mean Diameter (D3,2)

- Purpose: Surface-area weighted mean diameter; reflects the specific interfacial surface area per unit volume of the dispersed phase.

D3,2=i=1ndi3i=1ndi2

Where:

di — equivalent circular diameter of the i-th particle in the phase (μm);

n — total number of particles in the analyzed phase.

#### 4.2. De Brouckere Mean Diameter (D4,3)

- Purpose: Volume- or mass-weighted mean diameter; highly sensitive to the presence of large droplets, coarse air cells, or coalesced aggregates.

D4,3=i=1ndi4i=1ndi3

Where:

di — equivalent circular diameter of the i-th particle (μm);

n — total number of particles in the phase.

#### 4.3. Cumulative Distribution Percentiles (D10,D50,D90) and Relative Span

- Purpose: Quantifies the median particle size (D50) and the breadth (polydispersity) of the size distribution.

Span=D90-D10D50

Where:

D10,D50,D90 — 10th, 50th (median), and 90th percentile diameters of the number-based cumulative size distribution (μm);

Span — dimensionless relative polydispersity index.

### 5. Interfacial Adsorption Analytics (Pickering Stabilization)

Let B denote the set of user-verified air bubbles. A fat globule j is classified as interfacially adsorbed on bubble b∈B if its centroid xc,j,yc,j lies inside the binary mask Ωb of bubble b.

#### 5.1. 2D Bubble Surface Coverage (Θ2D,b and Θ2D,overall)

- Purpose: Quantifies the percentage of the projected air-bubble crater surface physically armored by adsorbed fat globules, eliminating double-counting of overlapping globule masks via spatial mask union.

Aads,b=Ωb∩j∈GbΩj⋅Rμm/px2, Θ2D,b=min100, Aads,bAb×100%

Θ2D,overall=min100, b∈BAads,bb∈BAb×100%

Where:

Ωb — binary pixel mask of air bubble b;

Gb — set of adsorbed fat globules whose centroids lie within Ωb;

Ωj — binary pixel mask of adsorbed fat globule j∈Gb;

Aads,b — net union area of adsorbed fat within bubble b (μm2);

Ab=Ωb⋅Rμm/px2 — projected area of bubble b (μm2);

Θ2D,b,Θ2D,overall — per-bubble and area-weighted overall interfacial surface coverage (%).

#### 5.2. Interfacial Packing Density (Ns,b)

- Purpose: Measures the number concentration of adsorbed fat globules per unit area of the air–water interface.

Ns,b=GbAb

Where:

Ns,b — interfacial packing density on bubble b (globules⋅μm-2);

Gb — count of adsorbed fat globules inside bubble b;

Ab — projected area of bubble b (μm2).

#### 5.3. Equatorial Rim Coverage (Θrim,b)

- Purpose: Quantifies fat coverage specifically within the equatorial perimeter ring of the bubble cross-section, compensating for SEM shadow drop-off at the bottom of deep craters.

An inward annular rim mask Ωrim,b of thickness wrim,px=max3, roundDref/Rμm/px is generated by morphological erosion:

Ωrim,b=Ωb∖Ωb⊖Kwrim,px, Θrim,b=Ωrim,b∩j∈GbΩjΩrim,b×100%

Where:

⊖ — morphological erosion operator using an elliptical structuring element K of radius wrim,px;

Dref — reference fat globule diameter (μm);

Θrim,b — percentage of the perimeter ring occupied by adsorbed fat (%).

#### 5.4. Protrusion Ratio (λj) and Apparent Three-Phase Contact Angle (θj)

- Purpose: Estimates the wetting/immersion depth of adsorbed fat globules at the air–water interface from top-down 2D projections using statistical reference scaling.

λj=Lc,jDref, θj=arcsinclipλj, 0.0, 1.0×180∘π

Where:

Lc,j — visible interfacial chord length of adsorbed globule j, taken as its fitted major axis length Lmajor,j (or deq,j if major axis is unavailable) (μm);

Dref — reference true diameter of intact fat globules (μm), selectable as the arithmetic mean diameter of free matrix fat (dmatrix), the Sauter mean diameter of matrix fat (D3,2matrix), or an externally calibrated constant;

λj — dimensionless chord-to-diameter protrusion ratio;

θj — apparent contact angle in degrees (θj∈0∘,90∘, where θj→90∘ denotes equatorial anchoring or interfacial spreading/coalescence where Lc,j≥Dref).

### 6. Matrix Partial Coalescence and Region Adjacency Graph (RAG) Topology

To evaluate the 3D fat crystal network in the continuous dispersion medium, PhasePix constructs an undirected Region Adjacency Graph G=V,E over all free matrix fat globules (phase_class=fat_particles) whose centroids lie strictly outside the cumulative air-bubble mask (Ωbubblesyc,i,xc,i=0) and within the user-selected Region of Interest (ROI).

#### 6.1. Inter-Globule Contact Criterion (Edge Definition)

- Purpose: Identifies physical sintering necks and contact bridges between partially coalesced fat globules i,j∈V×V.

An undirected edge eij=i,j∈E is established if and only if either the binary masks touch within a morphological dilation tolerance δpx OR their Euclidean centroid distance satisfies the proximity criterion:

i,j∈E⟺Ωi⊕Kδpx∩Ωj>0 ∨ dc,ij≤0.55deq,i+deq,j+δpx⋅Rμm/px

Where:

Ωi,Ωj — binary masks of matrix fat globules i and j;

⊕Kδpx — morphological dilation with an elliptical kernel of radius δpx (contact_dilation_px, default 3 px);

dc,ij=Rμm/pxxc,i-xc,j2+yc,i-yc,j2 — physical distance between centroids (μm);

deq,i,deq,j — equivalent diameters of globules i and j (μm).

#### 6.2. Coordination Number (Zi) and Topological Network Roles

- Purpose: Quantifies the number of direct physical contacts per fat globule and classifies globules into structural motifs within the colloidal network.

Zi=degi=j∈V:i,j∈E, Z=1Vi∈VZi

Topological Classification:

Zi=0: Isolated Monomer (free uncoalesced droplet);

Zi=1: Dimer / Chain End (terminal globule of a cluster);

Zi=2: Chain Link (linearly bridging globule);

Zi≥3: Network Branch Node (cross-linking junction of the 3D fat gel network).

- Derived Fractions:

Fcoal=i∈V:Zi≥1V×100%, Fbranch=i∈V:Zi≥3V×100%

Where:

Zi — coordination number of globule i;

Z — mean coordination number across the matrix ROI;

V — total number of analyzed matrix fat globules;

Fcoal — coalesced fat fraction (%);

Fbranch — fraction of network branching nodes (%).

#### 6.3. Connected Cluster Agglomeration Number (Nagg,m) and Equivalent Cluster Diameter (Dcluster,m)

- Purpose: Quantifies the size and subunit count of multi-globule partial-coalescence flocs/agglomerates extracted via Breadth-First/Depth-First Search (BFS/DFS) connected-components traversal of graph G.

For every connected component (cluster) Cm⊆V containing Cm≥2 globules:

Nagg,m=Cm, Acluster,m=i∈CmAi, Dcluster,m=4Acluster,mπ, Nagg=1Mcm=1McNagg,m

Where:

Cm — m-th multi-globule connected component (Cm≥2);

Nagg,m — number of primary fat globules in cluster m;

Nagg — mean aggregation number across all Mc multi-globule clusters;

Acluster,m — total projected area of cluster m (μm2);

Dcluster,m — area-equivalent diameter of the coalesced cluster (μm).