# SCIENTIFIC METHODOLOGY — LIGHT MICROSCOPY.

- Optical Micrograph Acquisition and Spatial Calibration

1.1. Optical Micrograph Acquisition and Spatial Calibration

Brightfield transmitted-light micrographs of the emulsions were analyzed using PhasePix (Quantitative Light Microscopy Emulsion Analyzer), a custom Python-based computer vision and colloidal morphometry platform. Spatial calibration (μm⋅px-1) was established using stage-micrometer-validated objective presets spanning 10× to 100× magnification (with and without 1.6× Optovar magnification, corresponding to spatial resolutions from 0.7210 μm⋅px-1 at 10× to 0.0467 μm⋅px-1 at 100×+1.6×). Raw 8-bit and 16-bit TIFF micrographs were normalized to linear luminance space [0, 255] for photometric evaluation, while calibrated scale bars were rendered directly onto exported micrographs according to the active objective coefficient.

1.2. Edge-Preserving Crystal Smoothing and Hierarchical Segmentation

- In transmitted-light micrographs of crystallized emulsions, internal fat crystal needles create dark refractive striations that can cause neural networks to erroneously split single globules into fragments.

- Pre-processing: If the crystallized profile is selected, the image undergoes edge-preserving smoothing (median filtering followed by bilateral filtering). This homogenizes the internal droplet texture while preserving the sharp outer refractive rims.

- Multi-Scale Segmentation: A two-pass hierarchical strategy is applied. The first pass evaluates the image at a large target diameter with strong smoothing to capture large macro-globules intact. The second pass evaluates the remaining matrix at a smaller diameter. Small candidate masks that significantly overlap with already-captured large globules are rejected as internal crystal shards.

1.3. Automated Brightfield Diagnostics (AI Parameter Advisor)

The brightfield diagnostic engine independently evaluates the structural geometry and the internal texture of the droplets:

- Polarity and Packing Assessment: By isolating inter-droplet dark ridges via black-hat filtering, the engine determines the packing fraction and recommends whether to invert the brightfield image (required for dilute dark-rimmed droplets, but disabled for tightly packed bright-body globules).

- Internal Crystal Detection: Striation density and Laplacian high-frequency energy are measured strictly inside the eroded cores of the droplets to detect internal fat crystals. High internal variance automatically recommends the application of edge-preserving smoothing.

- Target Scale Estimation: Touching droplets are temporarily split using a marker-controlled watershed based on distance-transform maxima. For liquid emulsions, the recommended AI target scale is derived from an area-weighted mean diameter. For highly polydisperse or crystallized systems, a Winsorized Sauter mean diameter (capped at the 85th percentile) is utilized to prevent a few large coalesced lobes from skewing the detection scale.

### 2. Optical Preprocessing and Deep Learning Droplet Segmentation

Automated droplet segmentation was performed using the Cellpose (cyto2) convolutional neural network architecture accelerated via Apple Silicon Metal Performance Shaders (MPS) / CUDA. To account for distinct optical signatures between liquid and crystalline lipid phases, two specialized preprocessing and inference pipelines were employed:

- Liquid-Fat Emulsion Pipeline (Homogeneous Brightfield Droplets): For emulsions containing liquid oil droplets characterized by bright refractive cores surrounded by dark diffraction halos against a bright background, bitwise intensity inversion (Iinv=255-I) was applied prior to neural network inference. The target network receptive field diameter (dAI,px) was scaled directly from the physical characteristic droplet diameter (dAI,μm) using the objective calibration factor (dAI,px=dAI,μm/pixel_to_um_ratio), ensuring accurate outer-rim boundary capture while extracting true grayscale intensities from the uninverted image.

- Crystallized-Fat Emulsion Pipeline (Edge-Preserving Smoothing & Multi-Scale Inference):

- For emulsions containing partially or fully crystallized fat globules exhibiting internal crystal striations, needle networks, and broad polydispersity, an edge-preserving cascade filter was applied prior to segmentation. Specifically, a median filter (kernel size 2s+1, where s∈1,4 denotes smoothing strength) followed by a bilateral filter

(d=9, σcolor=35s, σspace=7s)

homogenized internal crystal refraction lines while preserving sharp inter-droplet phase boundaries. To prevent over-segmentation of large crystalline globules into internal shards while retaining sensitivity to fine matrix droplets, a two-scale hierarchical inference was executed:

  - Coarse Pass (1.75×dAI,px, smoothing strength s+1): Segmented intact macro-globules without fracturing along internal crystal planes; external contours were hole-filled and retained if convexity (solidity) satisfied Si≥0.76.

  - Fine Pass (1.0×dAI,px, smoothing strength s): Segmented micro-droplets across the remaining continuous phase. Candidate detections exhibiting ≥25% spatial overlap with accepted macro-globules were discarded as internal crystal sub-fragments.

Across both pipelines, nested duplicate detections (artifacts) were removed via top-down area sorting whenever a smaller mask overlapped ≥60% with the hole-filled footprint of a larger parent droplet. Where necessary, challenging droplets were refined non-destructively using a hybrid Segment Anything Model (sam_vit_b_01ec64) bounding-box prompt paired with a 72-ray elliptical gradient rim snapper or periodic cubic spline interpolation (bc_type='periodic').

### 3. Morphological Filtering and Granulometric Quantification

Segmented objects were filtered by physical size bounds dmin,dmax (μm), circularity (Ci), and solidity / convexity (Si) to eliminate non-droplet optical artifacts, crystal shards, and inter-droplet Plateau border regions:

Ai=Apx,i⋅pixel_to_um_ratio2,  di=4Aiπ

Ci=4πApx,iPpx,i2≥Cmin,  Si=Apx,iAconvex hull,i≥Smin

For the verified droplet population (N), the Sauter surface-area weighted mean diameter (D3,2), De Brouckere volume-weighted mean diameter (D4,3), cumulative number-based percentiles (D10,D50,D90), and relative polydispersity Span were computed as:

D3,2=i=1Ndi3i=1Ndi2,  D4,3=i=1Ndi4i=1Ndi3,  Span=D90-D10D50

Particle size distributions (PSD) were constructed using fixed physical diameter intervals (Δd∈0.2,0.25,0.5,1.0,2.0 μm) aligned to exact multiples of Δd. Bar heights were normalized to the relative percentage of the droplet population

(Pk=nk/N×100%, Pk=100.0%)

and overlaid with a Gaussian Kernel Density Estimation (KDE) curve, enabling direct quantitative comparison across micrographs independent of total droplet count N.

### 4. Partial Coalescence and Droplet Contact Network (RAG) Topology

To quantify droplet flocculation, sintering neck formation, and partial coalescence in structured emulsions, a Region Adjacency Graph (RAG) G=V,E was constructed over all segmented fat droplets (V) within the analyzed region of interest. Two droplets i,j were connected by a contact bridge eij∈E if their binary masks intersected within a morphological dilation tolerance of δpx pixels (default: 3 px), or if their inter-centroid distance rij (μm) satisfied:

rij≤0.55di+dj+δpx⋅pixel_to_um_ratio

Each droplet was characterized by its coordination number (Zi=degvi), aspect ratio (ARi=amaj,i/amin,i), and solidity (Si), and categorized into four structural network motifs:

- Z=0: Isolated Monomer (unflocculated free droplet)

- Z=1: Dimer / Chain Terminal End

- Z=2: Linear Chain Link

- Z≥3: Network Branching Node (3D space-filling colloidal network junction)

Multi-droplet agglomerates (Nagg≥2) were identified via breadth-first connected-component traversal of G, yielding the Coalesced Droplet Fraction

(Φcoal=PZ≥1×100%),

Mean Coordination Number (Z), Branching Node Fraction

(PZ≥3×100%),

Mean Aggregation Number (Nagg), and Equivalent Cluster Diameter

(Dcluster=4Ak/π).