# Standard Operating Procedure: Quantitative Brightfield Light Microscopy Emulsion Analysis Using PhasePix

Applicable System: PhasePix — Quantitative Light Microscopy Emulsion Analyzer

Target Systems: Oil-in-Water (O/W) and Water-in-Oil (W/O) Emulsions, Liquid Fat Dispersions, and Partially/Fully Crystallized Fat Globule Networks

Imaging Modality: Transmitted-Light / Brightfield Optical Microscopy

Hardware Backend: Apple Silicon (MPS), NVIDIA CUDA, or CPU

## 1. Purpose and Scope

This Standard Operating Procedure (SOP) defines the sequential operational workflow, parameter selection criteria, and quality control (human-in-the-loop) verification steps required to calibrate, segment, and quantify emulsion fat droplets from brightfield light microscopy images using PhasePix. It covers droplet size distribution (PSD), Sauter surface mean diameter (D3,2), De Brouckere volume mean diameter (D4,3), polydispersity Span, and matrix partial coalescence network topology (Region Adjacency Graph, coordination number Z, and agglomerate size Nagg) across both smooth liquid-fat emulsions and textured crystallized-fat emulsions.

## 2. Pre-Analysis Preparation

- Image Acquisition & Format: Save raw optical micrographs in .tif, .tiff, .png, .jpg, or .bmp format (8-bit or 16-bit) without pre-burned camera overlays if possible.

- Image Placement: Place all light microscopy images into the dedicated images/light/ directory inside the PhasePix root folder (or upload them directly via the browser interface).

- Optical Metadata: Record the exact Objective Lens (10x to 100x) and Optovar (1.6x) combination used during microscope acquisition.

## 3. Step-by-Step Operational Sequence

### Step 1: Modality Selection, Image Loading, and Banner Crop (Sidebar Sections 0–2)

- In the left sidebar under 0. Imaging Modality, select 💡 Light Microscopy.

  - System Behavior: PhasePix automatically locks phase classification to single-phase Emulsion (Fat Droplets Analysis), hides irrelevant air-bubble/Pickering tabs, sets Footer Height to 0 px by default, and switches the local image directory to images/light/.

- Under 1. Image Input, select the image source method (From 'images/' folder or Upload via Browser) and load the target micrograph.

- Under 2. Footer Panel Exclusion, leave Optional Banner Crop (Light Mode) at 0 px for standard microscope camera frames. If your microscope software appends a bottom information bar, expand this section and increase Footer Height (pixels) in 5 px increments until the banner is excluded.

### Step 2: Spatial Calibration Verification (Sidebar Section 3)

- In 3. Spatial Calibration, select the corresponding laboratory-validated preset from the Microscope Objective Preset dropdown:

| Objective Lens Preset | Stage Micrometer Reference | Calibration Factor (μm/px) | Spatial Resolution (px/μm) | Typical Droplet Range Resolved |
| --- | --- | --- | --- | --- |
| 10x Objective | 10 μm=13.87 px | 0.72098 | 1.387 | 5.0-150.0 μm |
| 10x + 1.6x Objective | 10 μm=21.74 px | 0.45998 | 2.174 | 3.0-100.0 μm |
| 20x Objective | 10 μm=28.29 px | 0.35348 | 2.829 | 2.0-80.0 μm |
| 20x + 1.6x Objective | 10 μm=43.50 px | 0.22989 | 4.350 | 1.5-60.0 μm |
| 40x Objective (Default) | 10 μm=55.82 px | 0.17915 | 5.582 | 1.0-50.0 μm |
| 40x + 1.6x Objective | 10 μm=85.75 px | 0.11662 | 8.575 | 0.7-35.0 μm |
| 63x Objective | 10 μm=87.54 px | 0.11424 | 8.754 | 0.6-30.0 μm |
| 63x + 1.6x Objective | 10 μm=134.77 px | 0.07420 | 13.477 | 0.4-20.0 μm |
| 100x Objective | 10 μm=139.87 px | 0.07149 | 13.987 | 0.4-20.0 μm |
| 100x + 1.6x Objective | 10 μm=214.28 px | 0.04667 | 21.428 | 0.25-15.0 μm |

- Custom Recalibration & Persistence: If calibrating a new camera sensor or C-mount adapter, switch Calibration Input Method to Known Scale (µm / pixels), measure a stage micrometer division using the dashboard ruler (Step 3), enter Real Stage Micrometer / Scale Size (µm) and Measured Length (pixels), and click 💾 Save Current Value to Objective Preset to permanently update light_calibration_presets.json.

### Step 3: Rapid Feature Sizing via Dashboard Interactive Ruler (Tab: 📊 Interactive Dashboard)

Before configuring size boundaries or AI target scales for a new emulsion formulation or microscope objective:

- In the 📊 Interactive Dashboard tab, locate Section 1. Micrograph & Overlay and switch the display mode radio button from Segmentation Overlay to 📏 Interactive Ruler.

- Select the Ruler Base Image (Raw Micrograph or Segmentation Overlay).

- Click and drag directly across representative features (smallest fat droplet, typical matrix globule, and largest fat globule/coalesced cluster). The browser-native HTML5 caliper calculates physical dimensions with zero latency:

  - Measured Size: Exact physical length in micrometers (μm) and native pixels (px).

  - Suggested Min (80%) & Suggested Max (120%): Automatically computed 0.80× and 1.20× boundary guidelines.

- Use the Measurement History table below the micrograph (which automatically appears after the first line is drawn) to review multiple measurements, or use ↩️ Undo / 🗑️ Clear to reset lines.

### Step 4: Phase Classification & AI Detection Setup (Sidebar Sections 4–5)

In 4. Phase Classification, phase assignment is locked to Emulsion. Configure the physical size gates and 5. Cellpose AI Segmentation parameters according to the optical state of your lipid phase (Profile A: 💧 Liquid Fat vs. Profile B: ❄️ Crystallized Fat):

| Parameter | Profile A: 💧 Liquid Fat Emulsion (Smooth Globules) | Profile B: ❄️ Crystallized / Packed Fat Emulsion (Textured) | Operational Recommendation & Physical Rationale |
| --- | --- | --- | --- |
| Emulsion Droplet Profile | 💧 Liquid Fat (Smooth Globules) | ❄️ Crystallized Fat (Textured / Internal Crystals) | Switches preprocessing between smooth brightfield and edge-preserving crystal-smoothing + multi-scale detection. |
| Fat Particle Size Range (μm) | Min: 0.50 µm to Max: 40.0 µm | Min: 0.50 µm to Max: 50.0 – 60.0 µm | Rejects sub-micron Brownian debris below Min and flags oversized artifacts above Max as green Unclassified. |
| 🎯 Typical Droplet Diameter for AI (μm) | 3.0 – 6.0 µm (Default: 5.0 µm) | 8.0 – 14.0 µm (Default: 9.0 µm) | Converts physical μm to pixels via objective calibration. Increase if large crystallized globules split internally; decrease if small droplets are missed. |
| 🔍 Multi-Scale Globule Detection | Unchecked (False / Single-pass) | Checked (True) | Runs a 1.75× coarse pass (strong smoothing) to lock in large crystallized globules (12–25 μm) intact, then a 1.0× pass for small globules (2–7 μm), rejecting internal shards overlapping >25%. |
| ❄️ Internal Crystal Smoothing Strength | 0 (Off) | 2 – 3 (Range: 0–4, Default: 3) | Cascade of median (2s+1) and bilateral (σcolor=35s,σspace=7s) filters. Erases internal crystal needles inside globules while preserving sharp outer rims. |
| 💡 Invert Brightfield (Dark-Rim Droplets) | Checked (True) | Unchecked (False for packed) / Checked (True for dilute) | Bitwise image inversion (255-I). Keep ON when droplets have dark refractive halos on a bright background; keep OFF when tightly packed globules have bright bodies separated by thin dark contact lines. |
| 🛡️ Matrix Fat Min Solidity (Convexity) | 0.72 – 0.78 (Default: 0.75) | 0.82 – 0.88 (Default: 0.84) | Convexity ratio A/Ahull. True droplets are convex (S≥0.84); crescent crystal shards and inter-droplet Plateau border wedges are concave (S<0.80) and are automatically rejected. |
| 🟧 Matrix Fat Min Circularity | 0.65 – 0.78 (Default: 0.68) | 0.55 – 0.65 (Default: 0.60) | Shape factor 4πA/P2. Crystallized globules have slightly faceted perimeters (0.55–0.60); liquid droplets are nearly spherical (0.65–0.75). |
| Pretrained Model | cyto2 | cyto2 (or cyto3 for low contrast) | Generalist cytoplasmic network optimal for emulsion droplets (see Section 4A below for detailed selection rules). |
| flow_threshold | 0.70 (Default) | 0.65 – 0.75 (Default: 0.70) | Lower to 0.60 if noisy inter-droplet spaces produce false detections; raise to 0.80 if deformed crystalline globules are missed. |
| cellprob_threshold | -1.0 (Default) | -1.0 to -1.5 | Lower to -1.5 or -2.0 if faint out-of-focus droplets are missed; raise to -0.5 or 0.0 to reject background illumination gradients. |
| Enable CLAHE Preprocessing | Unchecked (False) | Unchecked (False) | Keep OFF in Light Microscopy by default: CLAHE amplifies brightfield refractive halos and exaggerates internal crystal striations. |

- Click 🔄 Apply Size Ranges & Classification after modifying size boundaries.

#### 4A. Foundation Model Selection Guidelines (Pretrained Model — Sidebar Section 5)

PhasePix supports three built-in Cellpose architectures plus custom-trained weights (Custom Model File...). Select the model manually according to the optical complexity of your sample:

| Model Architecture | Recommended Modality & Sample Conditions | Operational Rationale & Speed/Sensitivity Trade-off |
| --- | --- | --- |
| cyto2 (Default Baseline) | Standard Brightfield & Crystallized Emulsions: Moderate-to-high contrast micrographs (ΔIP95-P5≥65 GL in liquid mode; ≥75 GL in crystallized mode when smoothing strength ≥3 and multi-scale detection are active). | Classic generalist cytoplasmic network. Provides the fastest inference speed and clean boundary closure when droplet rims are well-defined or internal crystal lines are already suppressed by smoothing. |
| cyto3 (Super-Generalist) | Low-Contrast Brightfield & Faint Crystallized Emulsions: Low optical contrast micrographs (ΔIP95-P5<65 GL for liquid droplets, or <75 GL for heavily striated crystallized globules) where outer droplet halos are faint or blurred. | Trained on a massive multi-modality dataset. Recovers faint, out-of-focus, or complex textured droplet boundaries that cyto2 misses, at the cost of slightly longer inference time. |
| nuclei | Simple Monodisperse Spherical Dispersions: High-contrast, isolated, strictly spherical liquid droplets without internal crystal texture or touching boundaries. | Optimized for round, compact convex bodies; not recommended for faceted crystallized globules or densely packed emulsions. |
| Custom Model File... | Specialized Formulations: Upload laboratory-specific .pth weights trained via the PhasePix 🎓 Export for Cellpose Training (.tif) package. | Directly loads custom weights into models_cache/ for domain-specific emulsion segmentation. |

#### 4B. Automated Parameter Optimization via 🪄 AI Parameter Advisor (Sidebar Section 5)

Instead of manually guessing optical profiles, target droplet scales, and convexity thresholds, use the built-in 🪄 AI Parameter Advisor:

- Trigger Diagnostics: In the left sidebar under 🪄 AI Parameter Advisor, click 🪄 Analyze Image & Recommend Parameters.

- Non-Destructive Diagnostic Review: The advisor analyzes the active micrograph in physical micrometers (μm) without immediately altering your sidebar settings, opening an interactive review card (🪄 AI Microstructure Diagnostics & Recommended Parameters) at the top of the workspace:

  - For Light Microscopy: Reports measured Droplet ∅ (P5–P95), AI Target ∅ (μm), Crystal Striation Density / Packing Fraction, Optical Contrast (P95-P5 in GL), and the Recommended Optical Profile (❄️ Crystallized Fat vs. 💧 Liquid Fat). Automatically configures Typical Droplet Diameter for AI (µm) (using a Winsorized Sauter D3,2P85+P82 blend for crystallized systems or an area-weighted D2,1+P75 blend for liquid systems), Min/Max Fat Particle Size (µm), Multi-Scale Globule Detection, Internal Crystal Smoothing Strength (3 vs. 0), Invert Brightfield polarity, Matrix Fat Min Solidity (0.84 vs. 0.75), Matrix Fat Min Circularity (0.60 vs. 0.68), and selects the optimal foundation model (cyto2 vs. cyto3).

  - For SEM Micrographs (Cross-Modality Reference): Reports measured Pore ∅ (P10–P95), Matrix Globule ∅ (P10–P95), Optical Contrast (P95-P5), and Crater Shadow / Speck Score, configuring Single-Phase vs. Two-Pass detection, physical size gates, Deep-Scan Adsorbed Fat, and cyto2 vs. cyto3.

- Controlled Application & One-Click Recovery:

  - Review the 📊 Parameter Comparison & Scientific Rationale table (Current vs. Recommended values with physical explanations).

  - Click ✅ Apply Recommended Parameters to update all Section 4 and Section 5 controls simultaneously and re-run segmentation, or click ✖️ Dismiss to keep current settings.

  - Use ↩️ Undo Last Apply in the sidebar to revert to the exact parameter snapshot prior to applying recommendations, or 🔄 Reset to Defaults to restore canonical modality defaults.

- 📂 Parameter Presets (Save / Load Recipes): Expand 📂 Parameter Presets (Save / Load Recipes) in the sidebar to load validated laboratory recipes (e.g., 💧 Liquid Emulsion (Standard 40x), ❄️ Crystallized Fat Emulsion (Multi-Scale 40x/63x)) or save your current Section 4/5 configuration into parameter_presets.json for reproducible multi-batch analysis.

## Step 5: Human-in-the-Loop Verification & Manual Correction (Tab: 🖌️ Manual Correction)

- Switch to the 🖌️ Manual Correction tab. Collapse the left sidebar (<<) to inspect the canvas at 100% native 1:1 resolution.

- Identify any missed large crystallized globules, out-of-focus droplets, or touching doublets that require manual addition.

- Choose the appropriate Selection Method based on feature visibility:

  - 📦 AI Smart Box & 🧠 SAM Backbone Selection: Recommended for well-defined liquid droplets or crystallized globules. Drag a tight rectangle around the droplet and click ➕ Detect & Add from Box(es). When 📦 AI Smart Box is selected, a secondary dropdown 🧠 SAM Backbone (Smart Box AI) appears in the sticky toolbar; select the Segment Anything Model (SAM) Vision Transformer backbone based on boundary complexity and hardware memory:

    - vit_b — Base (375 MB, Instant) (Default): Recommended for routine manual additions of well-defined fat droplets. Loads instantaneously and provides rapid interactive box-to-boundary fitting.

    - vit_l — Large (1.25 GB, Balanced): Recommended when vit_b slightly under-segments touching droplets or faint refractive halos; offers a balanced trade-off between embedding computation time and edge fidelity.

    - vit_h — Huge (2.56 GB, Max Precision for Complex Edges): Recommended for workstations with ample RAM/VRAM (e.g., 32–48 GB unified memory) when segmenting heavily crystallized, faceted, or low-contrast brightfield fat globules.

  - 📐 Exact Smooth Curve by Points: Recommended for irregular, strongly coalesced, or partially occluded globules. Click 4–8 points sequentially along the true physical rim of the droplet and click ➕ Add Manual Droplet. PhasePix constructs a periodic cubic spline passing 100% through your clicked coordinates (0.0 px shift).

  - 🪄 1-Click Dark Pore Center: Recommended for rapid single-click addition of circular droplets. Click once inside the droplet center and click ➕ Add Droplet.

- Set Assign New Manual Objects As (fat_particles or Auto (by Min/Max Size)) prior to committing.

- Mandatory Persistence Step: Scroll to 💾 Manual Mask Persistence and click 💾 Save Manual Objects to Disk. This writes images/<filename>_manual_masks.json, protecting manual work against session resets.

### Step 6: Phase Cleaning, Visualization & PSD Binning (Sidebar Section 6 & Tab: 📊 Interactive Dashboard)

- Return to 📊 Interactive Dashboard and inspect green objects (Unclassified) on the overlay:

  - If Unclassified objects are genuine macro-globules or fine droplets slightly outside the nominal size range, click 🟧 Assign All Unclassified → Fat Droplets (or use the 📏 Group & Assign Unclassified by Size Range (µm) expander).

  - If Unclassified objects are dust, air artifacts, or crystal debris, click 🗑️ Delete All Unclassified.

  - To remove individual artifacts, check the Excluded box in the 2. Phase Classifications table and click Apply Changes.

- Visualization & Burn-in Scale Bar (Sidebar Section 6):

  - Toggle 🔢 Show Particle ID Numbers ON during quality inspection to cross-reference individual droplets with the metrics table, and OFF when exporting clean color-coded maps for publications or reports.

  - Enable Draw Scale Bar on Images, set the desired physical length (e.g., 10 µm, 20 µm, or 25 µm), choose a high-contrast color (White or Black for raw micrographs; Yellow or Red for overlays), and adjust Thickness.

- Comparable PSD Histogram Binning (3. Size Distribution):

  - Select the fixed physical histogram bar width 🟧 Fat Histogram Step (∆µm) (0.2, 0.25, 0.5, 1.0, or 2.0 µm; typically 0.25–0.50 µm for liquid emulsions and 0.50–1.00 µm for crystallized emulsions).

  - Always use the exact same Δμm when comparing different formulations so Y-axis percentages (Σ=100.0%) remain directly comparable regardless of total droplet count n. Expand 📊 View Exact Binned Percentage Table — Fat (Σ = 100%) to inspect and export interval counts and percentages.

### Step 7: Matrix Partial Coalescence & Flocculation Network Analysis (Tab: 🍇 Matrix Coalescence)

- Open the 🍇 Matrix Coalescence tab to evaluate droplet clustering, sintering necks, and 3D lipid network formation via the Region Adjacency Graph (RAG):

  - Z=0 (Blue): Isolated monomer (free uncoalesced droplet).

  - Z=1 (Green): Dimer / chain end (terminal globule of a cluster).

  - Z=2 (Yellow): Chain link (linearly bridging globule).

  - Z≥3 (Red): Network branch node (cross-linking junction of a 3D coalesced fat network).

- If illumination at the edges of the optical field of view is uneven or vignetted, check 🎯 Analyze Specific Verified Sub-Region (ROI Box) of Matrix and adjust the Horizontal/Vertical ROI Range (%) sliders to isolate a clean, evenly illuminated region.

- Set Inter-Globule Contact Bridge Tolerance (px) (default: 3 px):

  - Recommendation: Keep at 2–3 px for standard liquid emulsions (40x); increase to 3–5 px for high-magnification (63x/100x) or crystallized emulsions where outer refractive rims leave a slight pixel gap across sintered fat necks.

### Step 8: Data Export & Scientific Archiving (Tabs: 📊 Dashboard, 🔬 Metrics, 🍇 Coalescence, 💾 Export)

- Export the Segmented Overlay (PNG), Raw Micrograph (PNG), Fat Size Distribution Plot (PNG), Fat Particles Data (CSV), and Binned Percentage Table (CSV) from 📊 Interactive Dashboard.

- Open 🔬 Metrics and click 📥 Download Results Table (CSV) to export the granulometry summary table (D3,2,D4,3,D10,D50,D90, and Relative Span) alongside individual droplet morphometrics.

- Export the Overall Coalescence Summary (CSV), Coalescence Network Map (PNG), Z-Distribution Plot (PNG), Cluster Agglomerates Table (Nagg≥2, CSV), and ROI Particles (CSV) from 🍇 Matrix Coalescence.

- Open 💾 Reproducibility & Export and click 📥 Download Manifest (.txt) to archive the complete session metadata (timestamp, objective preset, μm/px calibration factor, optical profile, crystal smoothing strength, multi-scale flag, Cellpose/SAM model architectures, and final droplet counts) alongside your experimental records. Optionally click 🎓 Export for Cellpose Training (.tif) to save 16-bit mask packages for future custom model fine-tuning.