## Document Title: Quantitative Brightfield Light Microscopy Emulsion Analysis using PhasePix

- Software Module: PhasePix — Quantitative Light Microscopy Emulsion Analyzer

- Target Systems: Oil-in-Water (O/W) and Water-in-Oil (W/O) Emulsions, Liquid Fat Dispersions, and Partially/Fully Crystallized Fat Globule Networks

- Imaging Modality: Transmitted-Light / Brightfield Optical Microscopy

- Hardware Backend: Apple Silicon (MPS), NVIDIA CUDA, or CPU

### 1. Purpose & Scope

This Standard Operating Procedure (SOP) defines the standardized protocol for calibrating, segmenting, and quantifying emulsion fat droplets from brightfield light microscopy images using PhasePix. It covers droplet size distribution (PSD), Sauter surface mean diameter (D3,2), De Brouckere volume mean diameter (D4,3), polydispersity Span, and matrix partial coalescence network topology (Region Adjacency Graph, coordination number Z, and agglomerate size Nagg) across both smooth liquid-fat emulsions and textured crystallized-fat emulsions.

### 2. Image Preparation & Folder Structure

- Save raw optical micrographs in .tif, .tiff, .png, or .jpg format (8-bit or 16-bit) without pre-burned camera overlays if possible.

- Place all light microscopy images into the dedicated images/light/ folder inside the PhasePix directory (or upload them via the browser).

- Record the exact Objective Lens and Optovar (1.6x) combination used during acquisition.

### 3. Step-by-Step Operating Procedure

#### Step 1: Modality Selection & Image Loading

- In the left sidebar under 0. Imaging Modality, select 💡 Light Microscopy.

  - System behavior: PhasePix automatically locks phase classification to single-phase Emulsion, sets Footer Height to 0 px, and switches the local image directory to images/light/.

- Under 1. Image Input, select the target micrograph from images/light/ or upload it via the browser.

#### Step 2: Spatial Calibration (3. Spatial Calibration)

Select the corresponding laboratory-calibrated preset from the Microscope Objective Preset dropdown:

| Objective Lens Preset | Stage Micrometer Reference | Calibration Factor (μm/px) | Spatial Resolution (px/μm) | Typical Droplet Range Resolved |
| --- | --- | --- | --- | --- |
| 10x Objective | 10 μm=13.87 px | 0.72098 | 1.387 | 5.0-150.0 μm |
| 10x + 1.6x Objective | 10 μm=21.74 px | 0.45998 | 2.174 | 3.0-100.0 μm |
| 20x Objective | 10 μm=28.29 px | 0.35348 | 2.829 | 2.0-80.0 μm |
| 20x + 1.6x Objective | 10 μm=43.50 px | 0.22989 | 4.350 | 1.5-60.0 μm |
| 40x Objective (Default) | 10 μm=55.82 px | 0.17915 | 5.582 | 1.0-50.0 μm |
| 40x + 1.6x Objective | 10 μm=85.75 px | 0.11662 | 8.575 | 0.7-35.0 μm |
| 63x Objective | 10 μm=87.54 px | 0.11424 | 8.754 | 0.6-30.0 μm |
| 63x + 1.6x Objective | 10 μm=134.77 px | 0.07420 | 13.477 | 0.4-20.0 μm |
| 100x Objective | 10 μm=139.87 px | 0.07149 | 13.987 | 0.4-20.0 μm |
| 100x + 1.6x Objective | 10 μm=214.28 px | 0.04667 | 21.428 | 0.25-15.0 μm |

Note: If recalibrating a new camera or C-mount adapter, select Known Scale (µm / pixels), enter the stage micrometer measurement, and click 💾 Save Current Value to Objective Preset to update light_calibration_presets.json.

#### Step 3: Physical Size Boundaries (4. Phase Classification)

- Set Min Fat Particle Size (µm) (typically 0.50 µm to 1.00 µm to reject sub-micron Brownian noise/debris).

- Set Max Fat Particle Size (µm) (typically 35.0 µm to 50.0 µm).

- Click 🔄 Apply Size Ranges & Classification. (Important: these values are strictly in physical micrometers μm; any detected object outside this range is flagged green as Unclassified).

#### Step 4: Emulsion Optical Profile & AI Tuning (5. Cellpose AI Segmentation)

Select the profile matching the physical state of the lipid phase:

- Profile A: 💧 Liquid Fat (Smooth Globules)

  - Use for emulsions above the lipid melting point or liquid oils where droplets have smooth, featureless interiors and a dark refractive outer rim on a bright background.

  - Keep 💡 Invert Brightfield (Dark-Rim Droplets) ON (True).

  - Set 🎯 Typical Droplet Diameter for AI (µm) to the visual average droplet size (typically 3.0 – 6.0 µm).

  - Set 🛡️ Matrix Fat Min Solidity to 0.72 – 0.78.

- Profile B: ❄️ Crystallized Fat (Textured / Internal Crystals)

  - Use for emulsions containing crystalline fat (e.g., palm kernel oil, milk fat, cocoa butter, whipped/churned emulsions) where internal crystal needles/lamellae create dark striations inside globules and droplets are tightly packed.

  - Enable 🔍 Multi-Scale Globule Detection (Large + Small Globules) (True).

  - Set ❄️ Internal Crystal Smoothing Strength to 2 or 3 (wipes out internal crystal lines before AI inference while preserving outer droplet rims).

  - Set 🎯 Typical Droplet Diameter for AI (µm) to 8.0 – 14.0 µm.

  - Keep 💡 Invert Brightfield OFF (False) for densely packed globules with bright interiors and dark contact boundaries, or ON (True) for dilute dark-rimmed droplets.

  - Set 🛡️ Matrix Fat Min Solidity to 0.82 – 0.86 to automatically reject crescent-shaped crystal shards and inter-droplet Plateau border triangles.

#### Step 5: Visualization, Scale Bar & PSD Binning (6. Visualization Settings & Tab 1)

- Toggle 🔢 Show Particle ID Numbers ON during quality inspection to cross-reference individual droplets with the metrics table, and OFF when exporting clean maps for publications/presentations.

- Enable Draw Scale Bar on Images, set the desired physical length (e.g., 10 µm, 20 µm, or 25 µm), choose a high-contrast color (Red or Yellow for overlays; White or Black for raw micrographs), and download both the Segmented Overlay (PNG) and Raw Micrograph (PNG).

- Under 3. Size Distribution, select the fixed physical histogram step 🟧 Fat Histogram Step (∆µm) (0.2, 0.25, 0.5, 1.0, or 2.0 µm). Use the exact same Δμm when comparing different formulations so Y-axis percentages (=100%) are directly comparable.

### 4. Light Microscopy Parameter Reference & Troubleshooting Table

| Parameter | Liquid Fat Emulsion | Crystallized / Packed Fat Emulsion | Physical Role & Diagnostic Troubleshooting |
| --- | --- | --- | --- |
| Emulsion Droplet Profile | 💧 Liquid Fat (Smooth) | ❄️ Crystallized Fat (Textured) | Switches preprocessing between raw/inverted brightfield and edge-preserving crystal-smoothing + multi-scale detection. |
| 🎯 Typical Droplet Diameter for AI (μm) | 3.0 – 6.0 µm | 8.0 – 14.0 µm | Converts target μm to pixels via objective calibration so Cellpose scales droplets to its native 30 px receptive field. If large globules split into fragments: increase this value. If small droplets are missed: decrease this value. |
| 🔍 Multi-Scale Globule Detection | N/A (Single-pass) | ON (Checked) | Runs a 1.75× coarse pass (strong smoothing) to lock in large crystallized globules (12–25 µm) whole, then a 1.0× pass for small globules (2–7 µm), discarding internal shards overlapping >25%. |
| ❄️ Internal Crystal Smoothing Strength | 0 (Off) | 2 – 3 (Range: 0–4) | Cascade of median (2s+1) and bilateral (σcolor=35s,σspace=7s) filters. Erases internal crystal needles inside globules without blurring the outer droplet rim. Increase to 3–4 if internal crystal lines still cause over-segmentation. |
| 💡 Invert Brightfield (Dark-Rim Droplets) | ON (Checked) | OFF (Packed) / ON (Dilute) | Bitwise image inversion (255-I) for AI only. Keep ON when droplets have dark refractive halos on a white background. Turn OFF when packed globules have bright bodies separated by thin dark contact lines. |
| 🛡️ Matrix Fat Min Solidity (Convexity) | 0.72 – 0.78 | 0.82 – 0.88 (Default: 0.84) | Convexity ratio Area/Areahull. True droplets are convex (S≥0.84). Broken crystal caps, crescent shards, and inter-droplet background wedges are concave (S<0.80) and are automatically rejected. |
| 🟧 Matrix Fat Min Circularity | 0.65 – 0.78 | 0.55 – 0.65 (Default: 0.60) | Shape factor 4πA/P2. Crystallized globules often have slightly irregular/faceted perimeters, so keep around 0.55–0.60; liquid droplets are nearly spherical (0.65–0.75). |
| flow_threshold | 0.70 | 0.65 – 0.75 | Allowed Cellpose flow error. Lower to 0.60 if noisy inter-droplet spaces produce false detections; raise to 0.80 if deformed crystalline globules are missed. |
| cellprob_threshold | -1.0 | -1.0 to -1.5 | Foreground probability gate. Lower to -1.5 or -2.0 if faint out-of-focus droplets are missed; raise to -0.5 or 0.0 to reject background illumination gradients. |
| Enable CLAHE Preprocessing | OFF | OFF | Keep OFF in Light Microscopy by default. CLAHE amplifies sensor noise in brightfield backgrounds and exaggerates internal crystal striations. Only enable for extremely low-contrast unstained micrographs. |
| 🟧 Fat Histogram Step (Δμm) | 0.25 or 0.50 µm | 0.50 or 1.00 µm | Fixed physical bin width on the X-axis. Guarantees that Y-axis percentages sum to Σ=100.0% and remain directly comparable across images regardless of total droplet count n. |
| Contact Bridge Tolerance (px) | 2 – 3 px | 3 – 5 px | Used in 🍇 Matrix Coalescence. Defines the maximum pixel gap between droplet masks to register a partial-coalescence sintering neck or flocculation contact bridge. |

### 5. Manual Correction & Coalescence Network Verification

- Manual Droplet Addition (🖌️ Manual Correction): If a large out-of-focus or strongly crystallized globule is missed by the automated pass, switch to 🖌️ Manual Correction, select 📦 AI Smart Box (to drag a rectangle around the globule) or 📐 Exact Smooth Curve by Points (to click 4–6 points along its rim), and click ➕ Detect & Add from Box(es) / ➕ Add Manual Droplet. Click 💾 Save Manual Objects to Disk to store the masks in images/{stem}_manual_masks.json.

- Coalescence & Flocculation Analysis (🍇 Matrix Coalescence): Switch to 🍇 Matrix Coalescence to evaluate droplet clustering, sintering necks, and 3D network formation (Z=0 isolated monomers, Z=1 dimers, Z=2 chain links, Z≥3 branching nodes). Use the ROI Box toggle if illumination at the edges of the optical field of view is uneven.

# SCIENTIFIC METHODOLOGY — LIGHT MICROSCOPY (FOR PUBLICATIONS & REPORTS)

Below is a ready-to-use Materials & Methods section specifically written for Brightfield Light Microscopy emulsion analysis.

### 1. Optical Micrograph Acquisition and Spatial Calibration

Brightfield transmitted-light micrographs of the emulsions were analyzed using PhasePix (Quantitative Light Microscopy Emulsion Analyzer), a custom Python-based computer vision and colloidal morphometry platform. Spatial calibration (μm⋅px-1) was established using stage-micrometer-validated objective presets spanning 10× to 100× magnification (with and without 1.6× Optovar magnification, corresponding to spatial resolutions from 0.7210 μm⋅px-1 at 10× to 0.0467 μm⋅px-1 at 100×+1.6×). Raw 8-bit and 16-bit TIFF micrographs were normalized to linear luminance space [0, 255] for photometric evaluation, while calibrated scale bars were rendered directly onto exported micrographs according to the active objective coefficient.

### 2. Optical Preprocessing and Deep Learning Droplet Segmentation

Automated droplet segmentation was performed using the Cellpose (cyto2) convolutional neural network architecture accelerated via Apple Silicon Metal Performance Shaders (MPS) / CUDA. To account for distinct optical signatures between liquid and crystalline lipid phases, two specialized preprocessing and inference pipelines were employed:

- Liquid-Fat Emulsion Pipeline (Homogeneous Brightfield Droplets): For emulsions containing liquid oil droplets characterized by bright refractive cores surrounded by dark diffraction halos against a bright background, bitwise intensity inversion (Iinv=255-I) was applied prior to neural network inference. The target network receptive field diameter (dAI,px) was scaled directly from the physical characteristic droplet diameter (dAI,μm) using the objective calibration factor (dAI,px=dAI,μm/pixel_to_um_ratio), ensuring accurate outer-rim boundary capture while extracting true grayscale intensities from the uninverted image.

- Crystallized-Fat Emulsion Pipeline (Edge-Preserving Smoothing & Multi-Scale Inference):

For emulsions containing partially or fully crystallized fat globules exhibiting internal crystal striations, needle networks, and broad polydispersity, an edge-preserving cascade filter was applied prior to segmentation. Specifically, a median filter (kernel size 2s+1, where s∈1,4 denotes smoothing strength) followed by a bilateral filter (d=9, σcolor=35s, σspace=7s) homogenized internal crystal refraction lines while preserving sharp inter-droplet phase boundaries. To prevent over-segmentation of large crystalline globules into internal shards while retaining sensitivity to fine matrix droplets, a two-scale hierarchical inference was executed:

  - Coarse Pass (1.75×dAI,px, smoothing strength s+1): Segmented intact macro-globules without fracturing along internal crystal planes; external contours were hole-filled and retained if convexity (solidity) satisfied Si≥0.76.

  - Fine Pass (1.0×dAI,px, smoothing strength s): Segmented micro-droplets across the remaining continuous phase. Candidate detections exhibiting ≥25% spatial overlap with accepted macro-globules were discarded as internal crystal sub-fragments.

Across both pipelines, nested duplicate detections ("matryoshka" artifacts) were removed via top-down area sorting whenever a smaller mask overlapped ≥60% with the hole-filled footprint of a larger parent droplet. Where necessary, challenging droplets were refined non-destructively using a hybrid Segment Anything Model (sam_vit_b_01ec64) bounding-box prompt paired with a 72-ray elliptical gradient rim snapper or periodic cubic spline interpolation (bc_type='periodic').

### 3. Morphological Filtering and Granulometric Quantification

Segmented objects were filtered by physical size bounds dmin,dmax (μm), circularity (Ci), and solidity / convexity (Si) to eliminate non-droplet optical artifacts, crystal shards, and inter-droplet Plateau border regions:

Ai=Apx,i⋅pixel_to_um_ratio2,  di=4Aiπ

Ci=4πApx,iPpx,i2≥Cmin,  Si=Apx,iAconvex hull,i≥Smin

For the verified droplet population (N), the Sauter surface-area weighted mean diameter (D3,2), De Brouckere volume-weighted mean diameter (D4,3), cumulative number-based percentiles (D10,D50,D90), and relative polydispersity Span were computed as:

D3,2=i=1Ndi3i=1Ndi2,  D4,3=i=1Ndi4i=1Ndi3,  Span=D90-D10D50

Particle size distributions (PSD) were constructed using fixed physical diameter intervals (Δd∈0.2,0.25,0.5,1.0,2.0 μm) aligned to exact multiples of Δd. Bar heights were normalized to the relative percentage of the droplet population (Pk=nk/N×100%, Pk=100.0%) and overlaid with a Gaussian Kernel Density Estimation (KDE) curve, enabling direct quantitative comparison across micrographs independent of total droplet count N.

### 4. Partial Coalescence and Droplet Contact Network (RAG) Topology

To quantify droplet flocculation, sintering neck formation, and partial coalescence in structured emulsions, a Region Adjacency Graph (RAG) G=V,E was constructed over all segmented fat droplets (V) within the analyzed region of interest. Two droplets i,j were connected by a contact bridge eij∈E if their binary masks intersected within a morphological dilation tolerance of δpx pixels (default: 3 px), or if their inter-centroid distance rij (μm) satisfied:

rij≤0.55di+dj+δpx⋅pixel_to_um_ratio

Each droplet was characterized by its coordination number (Zi=degvi), aspect ratio (ARi=amaj,i/amin,i), and solidity (Si), and categorized into four structural network motifs:

- Z=0: Isolated Monomer (unflocculated free droplet)

- Z=1: Dimer / Chain Terminal End

- Z=2: Linear Chain Link

- Z≥3: Network Branching Node (3D space-filling colloidal network junction)

Multi-droplet agglomerates (Nagg≥2) were identified via breadth-first connected-component traversal of G, yielding the Coalesced Droplet Fraction (Φcoal=PZ≥1×100%), Mean Coordination Number (Z), Branching Node Fraction (PZ≥3×100%), Mean Aggregation Number (Nagg), and Equivalent Cluster Diameter (Dcluster=4Ak/π).

Як тобі така структура чисто під світлову мікроскопію? Чи потрібно додати в таблицю рекомендації під конкретні типи зразків, з якими ти зараз працюєш?