## AI Parameter Advisor для SEM and 🪄 AI Parameter Advisor та Multi-Scale Crystallized Segmentation for Light Microscopy

## AI Smart Box (SAM 5-Point + Box Prompt and 72-ray Elliptical Rim Snapper)

#### Automated Optical & Morphological Diagnostics (AI Parameter Advisor — SEM Mode)

To eliminate subjective parameter tuning, PhasePix includes a deterministic pre-segmentation diagnostic engine (_diagnose_sem_micrograph) that evaluates the footer-cropped grayscale micrograph I (downscaled so maxW,H≤900 px with rescaled calibration R'μm/px) across four physical stages:

- Global Photometric Contrast and Background Reference: Using a Gaussian-smoothed image Iblur=G5×5*I, global optical contrast and matrix background intensity are defined from intensity percentiles:

ΔIP95-P5=P95Iblur-P5Iblur, Ibg=P85Iblur

- Dark Crater Population & Shadow Depth Quantification: Candidate dark pores are isolated at threshold Tdark=P22Iblur (or minTOtsu,P22 when P22>medianIblur-10), cleaned via morphological opening (3×3) and closing (5×5), and filtered by circularity (Φk≥0.35) and minimum shadow depth:

ΔIshadow,k=Ibg-μI,k≥6.0 GL, ΔIshadow,med=mediankΔIshadow,k

where μI,k and σI,k are the mean and standard deviation of pixel intensities inside pore k. From the resulting pore diameter distribution dair,k, the 10th, 50th, and 95th percentiles (P10,air,P50,air,P95,air) are extracted.

- Fine Matrix Globule Isolation via White Top-Hat Filtering: Bright sub-3 μm matrix fat globules are isolated using morphological white top-hat filtering with an elliptical structuring element KLth of diameter Lth=clipround3.0/R'μm/px, 5, 41:

Itophat=Iblur-Iblur∘KLth, Ωfat,cand=Itophat≥max6.0, P99Itophat

Connected components yield the fine globule diameter distribution dfat,m and percentiles P10,fat,P50,fat,P95,fat.

- Internal Crater Speckle Detection & Automated Parameter Mapping: To detect interfacially adsorbed fat globules hidden within dark bubble craters, the advisor evaluates internal crater variance and top-hat speck density inside Ωdark:

ρspeck=p∈Ωdark:Itophatp≥max8.0,0.5Tfat|Ωdark|, Speckint=Imediandair,k≥2μmσI,k≥14.0 ∨ ρspeck≥0.02

Recommended parameters are then derived deterministically:

  - Physical Size Gates:

dmin,air=roundlabmax0.5, 0.70 P10,air, dmax,air=roundlabmaxdmin,air+1.0, 1.30 P95,air

dmin,fat=roundlabmax0.1, 0.60 P10,fat, dmax,fat=roundlabmaxdmin,fat+0.2, 1.40 P95,fat

  - Neural Thresholds & Foundation Model Selection:

τflow=0.80,if ΔIP95-P5<45 ∨ ΔIshadow,med>750.75,if ΔIP95-P5<65 ∨ ΔIshadow,med>550.70,otherwise, τcellprob=-1.5,if ΔIP95-P5<65 ∨ ΔIshadow,med>55-1.0,otherwise

Modelrec=cyto3,if ΔIP95-P5<65 ∨ ΔIshadow,med>55 ∨ Speckint=1cyto2,otherwise

### Edge-Preserving Crystal Smoothing, Multi-Scale Hierarchical Segmentation та AI Parameter Advisor (light mycroscopy)

#### Edge-Preserving Internal Crystal Smoothing and Hierarchical Multi-Scale Segmentation

In transmitted-light micrographs of crystallized fat emulsions, internal fat crystal needles create dark refraction striations inside bright globules. Standard segmentation misinterprets these internal striations as droplet boundaries, shattering single macro-globules into multiple crescent shards. PhasePix resolves this via a three-stage optical pipeline:

- Edge-Preserving Internal Crystal Smoothing (smooth_internal_crystals): Given smoothing strength s∈1,2,3,4, the 8-bit micrograph I is first filtered with a median kernel of size kmed=2s+1 to erase thin internal crystal needles, followed by a bilateral filter that homogenizes droplet interiors while preserving sharp outer refractive rims:

Imed=MedianBlurI, 2s+1, Ismooth=BilateralFilterImed, d=9, σcolor=35s, σspace=7s

- Two-Scale Hierarchical Crystallized Globule Segmentation (cached_segment_crystallized_multiscale): When Multi-Scale Globule Detection is enabled at base target diameter dbase,px=clipdtarget,μm/Rμm/px, 12, 220, two complementary Cellpose passes are executed:

  - Pass A (Coarse Macro-Globule Pass): Executed on a strongly smoothed image (slarge=maxs,2+1) at an expanded scale dlarge,px=clip1.75 dbase,px, 30, 240. Only candidate masks with area ≥36 px2 and convexity (solidity) Sk=Ak/Ahull,k≥0.76 are retained, flood-filled into solid footprints Ωocc, and assigned priority labels.

  - Pass B (Fine Interstitial Pass & Shard Rejection): Executed at standard scale dsmall,px=clipdbase,px, 14, 180 with smoothing strength maxs,1. Any fine candidate mask Ωsmall,m whose overlap fraction with the already occupied macro-globule canvas satisfies:

|Ωsmall,m∩Ωocc||Ωsmall,m|≥0.25

is discarded as an internal crystal shard; non-overlapping regions (|Ωsmall,m∖Ωocc|≥12 px2) are merged as intact small droplets.

#### Automated Optical & Morphological Diagnostics (AI Parameter Advisor — Light Microscopy Mode)

The brightfield diagnostic engine (_diagnose_light_microscopy) decouples outer droplet geometry from internal crystal texture using dual-view analysis:

- Macro-Droplet Footprint & Eroded Interior Core Extraction: On a crystal-smoothed copy (s=3), inter-droplet dark contact ridges are isolated via black-hat filtering (Kridge=max5,round1.2/R'μm/px|1)) and subtracted from a morphologically closed Otsu mask to determine field packing fraction ϕpack and optical polarity (bright-body globules vs. dark-rimmed dilute droplets). Each macro-droplet component c with maximum distance-transform radius rmax,c is eroded by an elliptical kernel of radius rerode,c=max3,round0.45 rmax,c) to isolate the pure droplet interior mask Ωint (strictly excluding outer refractive halos).

- Internal Crystal Striation Physics: On the unsmoothed image I, black-hat striation density ρstriat, trimmed interior intensity standard deviation σint (10th–90th percentile), and Laplacian high-frequency energy ELap are evaluated strictly inside Ωint:

ρstriat=p∈Ωint:BlackHat7×7Ip>12Ωint, Crystallized=Iρstriat≥0.035 ∨ σint>18.0 ∨ ϕpack>0.55∧ELap≥6.0

- Marker-Controlled Watershed Sizing & Winsorized Sauter Target Scale: Touching droplets on the smoothed size mask are split via marker-controlled watershed seeded by distance-transform local maxima separated by at least Δpeak=max6,round1.5/R'μm/px) px, yielding droplet diameters di and percentiles P5,P50,P75,P82,P90,P95,P98.

  - For Crystallized or Polydisperse Emulsions (P90/P50>1.6): To prevent a few coalesced lobes from inflating the scale while ensuring the 1.0× pass never cuts inside 15–24 μm macro-globules, diameters are Winsorized at P85 (d'i=mindi,P85) to compute a robust Sauter mean D3,2P85, blended 50/50 with P82:

D3,2P85=d'i3d'i2, dtarget,μm=clipround0.50.5 D3,2P85+0.5 P82, 3.0, 22.0

(enforcing dtarget,μm≥8.0 μm whenever P95≥11.0 μm).

  - For Liquid Emulsions: Uses the 50/50 blend of the area-weighted mean D2,1=di2/di and P75:

dtarget,μm=clipround0.50.5 D2,1+0.5 P75, 3.0, 22.0

  - Size Gates:

dmin,fat=max0.40, min1.0, 0.65 P5, dmax,fat=max25.0, 1.6 P98, 3.2 dtarget,μm

Contour Regularization and Human-in-the-Loop Curve Fitting)

#### Interactive Box-Prompted Segmentation (SAM 5-Point Prompt & Elliptical Rim Snapper)

When a user draws a bounding box B=x1,y1,x2,y2 of width WB=x2-x1 and height HB=y2-y1 in 📦 AI Smart Box mode (predict_interactive_box_mask), PhasePix executes a two-tier boundary solver:

- Tier A — Segment Anything Model (SAM) with Box + 5-Point Prompting: Using the selected Vision Transformer backbone (MSAM∈vit_b,vit_l,vit_h), image embeddings are computed on a CLAHE-enhanced RGB representation. Because the user box tightly frames a single particle/pore, its geometric center pc=x1+x2/2,y1+y2/2 is passed as a positive foreground prompt (l0=1), while four inset corner coordinates (δx=max2, 0.04WB, δy=max2, 0.04HB) are passed simultaneously as negative background prompts (l1..4=0) alongside box B. Among the predicted multimask candidates Ωm, a candidate is accepted if and only if:

  - It covers the box center (pc∈Ωm) and occupies an area ratio ρm=|Ωm|/WBHB∈0.30,0.88;

  - It does not touch all four inset corners (preventing degenerate rectangular box masks);

  - It maximizes the composite quality score Jm=sSAM,m+0.15 ρm. Accepted masks are hole-filled, morphologically opened with a 5×5 ellipse to sever thin necks to adjacent droplets, and regularized via Discrete Fourier smoothing.

- Tier B — Scale-Normalized Cellpose ROI & 72-Ray Elliptical Rim Snapper Fallback: If SAM is unavailable or rejects an ambiguous box, Cellpose evaluates a padded ROI at scale dbox=max0.85minWB,HB, 8.0 px (τflow=0.8,τcellprob=-1.8) to generate a boundary hint map Hedge. A 72-ray polar search is then cast from pc at angles θk=2πk/72. For each ray, the inscribed ellipse radius is:

rellθk=rxryrycosθk2+rxsinθk2, rx=WB2, ry=HB2

Searching strictly within the annular corridor r∈0.62 rellθk, 1.05 rellθk, the optimal rim radius r*θk maximizes the regularized Sobel gradient objective:

r*θk=argmaxr∇ICLAHExr,yrnorm+βedge IHedgexr,yr>0-0.45 r-0.92 rellθk

where βedge=40.0. The radial sequence r*θk is smoothed with a circular median filter (window size 5) and converted into a closed Fourier contour.

#### Single-Click Multi-Scale Seed Growing (1-Click Center Mode)

When a single seed point ps=xs,ys is clicked (auto_grow_dark_bubble_contour), PhasePix first evaluates a wide local window around ps using Cellpose across a geometric progression of diameters drun∈28,55,100,160 px on both direct and inverted CLAHE polarities (τflow=0.8,τcellprob=-1.8). The first scale that isolates a valid object covering ps returns its Fourier-regularized contour (Kh=3 if Φ≥0.72, else Kh=5). If neural segmentation finds no closed boundary, a 36-ray radial fallback locates the first peak of the absolute radial intensity derivative ∂Iblur/∂r along each ray, applies outlier rejection (rk-medianr≤0.35 medianr), circular median filtering, parametric ellipse fitting, and Kh=3 Fourier smoothing.